DY-H8080XR

DYMEC – Hardened, Industrial 8 port Gigabit Flat PoE Switch,

8 x 10/100/1000 TX, 30 Watt PSE, 2KV Surge Immunity / Isolation

Supports power 1, power 2, and power din (for external power adapter),

Input Voltage: 48 ~ 56 Volts DC, Operating Temperature: -40°C to +75°C

Din rail mount

Wall mount

Cabinet

H8080X

Introduction

This Industrial 8 port POE switch is designed for IP surveillance, traffic monitoring, industrial applications and for a broad range of other applications. The unique flat design is perfect for space-limited environments, such as a shallow cabinet or a small enclosure. It is equipped with 8 x 30W PSE ports, and can deliver up to 200 Watts of power to your PD devices. It can be used as a stand-alone device for buses, trucks, and other vehicles for surveillance purposes. It has been rigorously tested for your security, transportation and telco applications.

Specifications

| IEEE Standard | IEEE 802.3 10Base-T Ethernet IEEE 802.3u 100Base-TX Fast Ethernet IEEE 802.3ab 1000Base-T Gigabit Ethernet IEEE 802.3x Flow Control and Back Pressure, IEEE 802.3af for POE IEEE 802.3at for POE+ |
| --- | --- |
| Switch Architecture | Back-plane (Switching Fabric): 17Gbps |
| Data Processing | Store and Forward |
| Flow Control | IEEE 802.3x Flow Control and Back Pressure |
| Jumbo Frame | 9KB |
| MAC address Table Size | 4K |
| Packet Buffer Size | 1Mbits |
| Network Connector | 8 x RJ-45 10/100/1000BaseT(X) auto negotiation, Auto MDI/MDI-X function, Full/Half duplex 8 x Gigabit POE+ 802.3at/af PSE port, 30 Watts per port |
| Network Cable | UTP/STP above Cat.5e Cable |
|  | EIA/TIA-568 10-ohm (100m) |
| Protocol | CSMA/CD |
| LED | PW1 (Green): ON－Power 1 is detected PW2 (Green): ON－Power 2 is detected PW3 (Green): ON－Power 3 is detected for Power DIN RLY (Amber): ON－Connect only PW1 or PW2 OFF－Both PW1 and PW2 are connected |
|  | TX/RJ-45 port: LNK (Link/Active) (Green): ON－TX port is detected Flashing－TX data is transmitting/receiving POE (Amber): ON－PD is detected and PSE is activated OFF－PD is not detected |
| Reserve polarity protection | Yes |
| Overload current protection | Yes |
| Power Supply | Redundant Dual DC 48 ~ 56 Volt Power Inputs |
| Power Consumption | 5.76 Watts @ 48 Volts DC full load, Without POE |

| Alarm Relay Contact |  | Relay outputs - current carrying capacity of 1 AMP @ 24 Volts DC, Relay in open circuit mode when 2 powers are connected. in short circuit mode when only one power supply is connected |
| --- | --- | --- |
| POE power |  | POE power per port 30 Watts. Maximum total power 240 Watts with 56VDC input, Supports IEEE802.3af/at |
| Removable Terminal Block |  | Provides 2 Redundant power inputs, Alarm relay contact ,6 Pin Wire range: 0.34mm^2 to 2.5mm^2 Solid wire (AWG):12-24/14-22 Stranded wire(AWG): 12-24/14-22 Torque:5lb-In/0.5Nm/0.56Nm Wire Strip length: 7-8mm |
| Operating Temperature |  | -40°C to +85°C |
| Operating Humidity |  | 5% to 95% (Non-condensing) |
| Storage Temperature |  | -40°C to 85°C |
| MTBF (mean time between failure) |  | >525,000 hours ( MIL-HDBK-217F) at 25°C |
| Housing |  | Military Grade with Aluminum, IP30 Protection |
| Case Dimension |  | 150 x 145 x 26.7 mm (L x W x D) 198 x 145 x 26.7 mm (L x W x D) (with brackets) |
| Weight |  | 0.75kg (with brackets) |
| Installation |  | DIN Rail Mount or Wall Mount |
| Certifications: |  |  |
| Safety | LVD(EN60950-1) |  |
| EMC | CE, FCC, EN 55032/24, NEMA TS-2 |  |
| EMI | CISPR 32, FCC Part 15B Class A |  |
| EMS | IEC 61000-4-2 ESD: Contact: 6KV; Air: 8KV IEC 61000-4-4 EFT: Power: 2KV; Signal: 2KV IEC 61000-4-5 Surge: Power: 2KV; Signal: 2KV |  |
| Vibration | EN 60068-2-6 |  |
| Shock | EN 60068-2-27 |  |
| Free Fall | EN 60068-2-32 |  |

Housing Dimensions (mm)