Safety

UL 60950-1

DY-HI8011-9bt - Hardened 90-Watt. Gigabit, IEEE 802.3bt, POE Injector

1 x 95 Watt Gigabit TX PSE + 1 x Gigabit TX, 48-56 Volts DC input voltage,

Operating temp.: -40°C to +75°C

Features:

- Safety UL60950-1 Certified

- Rugged Military Grade Aluminum Housing

- Deliver ultrahigh power up to 95Watts

- Supports Redundant Power Inputs

- Working in extreme environment -40 to +85°C

DY-HI8011-90bt

Overview

This ultrahigh power hardened Gigabit POE Injector is equipped with 1 x 90-Watt Gigabit PSE port to power up to 90 Watts PD devices. Used for camera, WiFi AP, or LoRa applications. This unit was made especially for IP surveillance, traffic monitoring, outdoor cameras and for a broad range of power applications. It can tolerate -40°C to +75°C in harsh environments.

Specifications

| IEEE Standard | IEEE 802.3 10Base-T Ethernet IEEE 802.3u 100Base-TX Fast Ethernet IEEE 802.3ab 1000Base-T Gigabit Ethernet IEEE 802.3 af/at/bt |
| --- | --- |
| Network Connector | 1xRJ-45 10/100/1000BaseT(X) Data 1xRJ-45 10/100/1000BaseT(X) Data with POE Output Power |
| Network Cable | UTP/STP Cat.5e or above Cable |
|  | EIA/TIA-568 10-ohm (100m) |
| Protocol | CSMA/CD |
| LED | PW1 (Green): ON－Power is detected PW2 (Green): ON－Power is detected Mode A (Green): For End-Span PoE power 1,2,3,6 Mode B (Green): Reserved |
| POE Pin Assignment | Pin 1 (V-), 2 (V-), 3 (V+), 6 (V+) Pin 4 (V+), 5 (V+), 7 (V-), 8 (V-) |
| Reverse polarity protection | Present |
| Overload current protection | Present |
| Power Supply | 2 Redundant input power sources 48V-57V VDC Power Input |
| Power Consumption | 1 Watt @48 VDC Without POE |
| POE power | Maximum POE power 95watts at 56VDC input |
| Removable Terminal Block | Provide 4 pin terminal block Wire range: 0.34mm^2 to 2.5mm^2 Solid wire (AWG):12-24/14-22 Stranded wire(AWG): 12-24/14-22 Torque:5lb-In/0.5Nm/0.56Nm Wire Strip length: 7-8mm |
| Operating Temperature | -40°C to 85°C |
| Operating Humidity | 5% to 95% (Non-condensing) |
| Storage Temperature | -40°C to 75°C |
| MTBF (mean time between failure) | >5000,000 hours ( MIL-HDBK-217F) at 25°C |
| Housing | Rugged Aluminum, IP30 Protection |
| Case Dimension (L X W X D)mm | 103.5 x 32 x 81.5 mm (L x W x D) |
| Installation mounting | DIN Rail and Wall Mount options included |

| Certifications: |  |
| --- | --- |
| Safety | UL 60950-1 |
| Safety | IEC EN60950-1 |
| EMC/EMS | CE, FCC, VCCI |
| EN 60068-2-6 | Vibration |
| EN 60068-2-27 | Shock |
| EN 60068-2-32 | Free Fall |