KY-3170™EM4-Pbt / DYMEC 3170bt™ Series

6 x 4 Port PSE, Managed, Gigabit, Power over Ethernet (IEEE 802.3 af / at, bt), 30 / 60 / 95 - Watt, Temperature Hardened, ITS, Industrial, Input File, Ethernet Switch – Low Power / NSA Endpoint Certified with ERPSv2 Redundant Fiber Technology

|  | - Security Certified – IEC 62443 Compliant - 2 x 10/100/1000 - (60 / 95-Watt bt) PSE Ports - 4 x Gigabit Ethernet (30 Watt) PSE Ports - 4 x 1000 Mbps SFP Slot (Mini Gbic) / ERPS Multi Ring - Power over Ethernet Plus / IEEE 802.3 af/at (30 Watt) - New IEEE 802.3bt 60 / 95 Watt PoE++ standard - NSA Network Endpoint Guideline Certified for Console Port Shutdown and Chassis Full Security - Built for harsh environments and wide temperature range applications / Edge Computing - RSTP, MSTP, ERPS, LACP for Network Redundancy - QoS CoS/DSCP to increase determinism - Input File / Low Power Industrial Ethernet Switch - Supports IGMP Snooping v1/v2/v3, up to 1024 groups - DHCP Relay Option 82 IP address assignment - SNMP v1/v2c/v3 for network management - Supports Storm Protection / Broadcast Control - USB storage for configuration-free replacement - Supports industrial automation protocols Ethernet/IP & Modbus TCP - Secure File Transfer Protocol (SFTP) & Trivial File Transfer Protocol (TFTP) - Supports PoE PD Alive Check Function for PoE Device Power Recycle |
| --- | --- |
| Specifications |  |

| Technology |  |
| --- | --- |
| Standards | IEEE 802.3 10BaseT IEEE 802.3u 100BaseTX IEEE 802.3ab 1000BaseT IEEE 802.3z 1000BaseX IEEE 802.3x Flow Control IEEE 802.3af Power over Ethernet IEEE 802.3at PoE+ IEEE 802.3bt PoE++ IEEE 802.3ad/802.1AX LACP IEEE 802.1D Spanning Tree Protocol IEEE 802.1w Rapid STP IEEE 802.1s Multiple STP ITU-T G.8032 / Y.1344 ERPS IEEE 802.1Q VLAN Tagging IEEE 802.1ad Stacked VLANs IEEE 802.1p Class of Service IEEE 802.1X Network Authentication IEEE 802.1AB LLDP |
| Processing Type | Store and Forward |
| Switch Properties |  |
| Priority Queues | 8 Queues |
| Jumbo Frame | 9216 bytes |
| MAC Table Size | 16K |
| VLAN Table Size | 4094 |
| IGMP Group | 1023 |
| Packet Buffer | 12Mbits |
| Software Features |  |
| Network Redundancy | RSTP, MSTP, G.8032 ERPSv2, Port Trunking with LACP |
| Security | 802.1X, RADIUS, TACACS+, SSL, SSH, ACL / Security ACL Package |
| Management | IPv4, IPv6, Web, Telnet, Serial Console, CLI, SFTP, SNMP v1/v2c/v3, uPnP |
| Diagnostics | RMON, SNMP Inform/Trap, Syslog, SMTP, Port Mirroring, LLDP, DDMI, Cable Diagnostic* |
| Configuration | DHCP Server/Client, Relay Option 82, TFTP, SFTP, USB |
| Filter | 802.1Q VLAN, Q-in-Q, IGMP Snooping v1/v2/v3, IGMP Querier, Independent Vlan Learning |
| Time Sync. | NTP |
| QoS | CoS, DSCP, WRR/SPQ Queuing, Storm Protection, Broadcast Control |
| Industrial Profiles | Ethernet/IP, Modbus TCP |
| Switch Interfaces |  |
| RJ45 Port | 6x 10/100/1000T(x) with 4x PoE+ and 2x PoE++, auto negotiation speed duplex mode, auto MDI/MDI-X |
| Fiber Port | 4 x 1000 Mbps Fx SFP Slots |
| PoE Pin Out | V+, V+, V-, V-, for pin 1, 2, 3, 6, End-span, MDI Mode A V+, V+, V-, V-, for pin 4, 5, 7, 8, End-span, MDI Mode B 4-pair mode for 802.3bt |
| LED Indicators | Per unit: PWR1, PWR2, Fault, Ring Master, Ring State Ports: Link/Active with highest speed (Green), low speed (Amber) PoE: Output Power |
| Button | Multiple functions reset button |
| Serial Console | 1 x RS232 in RJ45 connector with console cable, 115.2Kbps, 8N1 |
| Storage | 1 x USB 2.0 storage for firmware update, configuration backup, restore, boot up and syslog |
| Power Requirements |  |
| Operation Voltage | 12 ~ 24 ~48 Volts DC |
| Connection | PCB Golden Finger |
| Power Consumption _______________ PoE Power Budget | 0.82A@24 Volts DC without PD’s Consumption _______________________________ 48~57 Volts DC /300 Watts Maximum for Total PD Consumption. Maximum 30 / 60 95 Watts per PoE Port |
| Protection | Overload Current Protected, Reverse Polarity Protected |
| Mechanical Construction |  |
| Enclosure | Metal / Military - Structural Grade |
| Protection Class | IP30 |
| Dimensions | 58 x 114 x 205 mm (W x H x D) |
| Weight | 0.87 kg |
| Mounting | Input File / Rack Mount with Rail Slots |
| Environmental Limits |  |
| Operating Temperature | Extended: -40°C ~ 85°C (-40°F ~ 185°F) |
| Storage Temperature | -40°C ~ 85°C (40°F ~ 185°F) |
| Ambient Relative Humidity | 5 to 95%, (Non-Condensing) |
| Regulatory Approvals |  |
| Altitude Certified GB4943.1-2011 | 0 ~ 5000 Meters |
| EMI | FCC Part 15 Subpart B Class A CE EN 55032 Class A |
| EMS | IEC61000-4-2 (ESD) IEC61000-4-3 (RS) IEC61000-4-4 (EFT) IEC61000-4-5 (Surge) IEC61000-4-6 (CS) IEC61000-4-8 (Magnetic Field) |
| Free Fall | IEC60068-2-32 |
| Security | IEC62443 |
| Shock | IEC60068-2-27 |
| Vibration | IEC60068-2-6 |
| Green | RoHS Compliant |
| Certifications | UL 60950-1/62368 / EN50155 IEC 61000-6-2 / NEMA TS-2 IEC 61000-6-4, IEEE, ITUT EN 50121-4 |
| MTBF MIL-HDBK-217 IEC 62380 | >171,000 hours |
| Warranty TAA Compliant | 5 Years / Limited Lifetime TAA Certified Compliant |

Note from DYMEC Development In a continuing effort of improvement and updated technology, product specifications are subject to change without notice.

Dimensions (unit = mm)

DYMEC 3170 Supports: 2 & 4 Wire LONG RANGE ETHERNET / VDSL2 SFP Modem

With long reach coverage, high noise immunity, and compact board size, DYMEC KY-Pro-180-CRD Ruggedized SFP modem is an ideal upgrade for your Enterprise or Industry applications. Used in Pairs.

##### VDSL2 – Long Range Ethernet Application (KY-Pro-180-CRD)

DYMEC KY-SFP-T

Copper SFP Transceiver – Used to Add Additional RJ-45 Copper Ports to your KY-3170 Series Switch

Easy to Install & Configure – GUI Selectable

| Ordering Information |  |
| --- | --- |
| KY-3170™EM4-Pbt DYMEC 3170bt™ Series | 6+4 Port, Industrial, Managed, PoE+, Gigabit Ethernet ITS Switch - 6 x 10/100/1000T(x) PSE + 4 x 1000 SFP Slots / ERPS - Operating Temperature: -40°C ~ 85°C (-40°F ~ 185°F) - 400 Watts PoE Maximum NSA Guideline Certified for Console Port Shutdown and Chassis Full Security Made in USA |

| KY-3170™EM4-Pbt / 2 DYMEC 3170bt™ Series | 8+2 Port, Industrial, Managed, PoE+, Gigabit Ethernet ITS Switch (Configured 8 Copper RJ45 Ports) - 6 x 10/100/1000T(x) PSE + 2 x 10/100/1000 Copper SFP + 2 x 1000 SFP Slot / ERPS - Operating Temperature: -40°C ~ 85°C (-40°F ~ 185°F) - 400 Watts PoE Maximum NSA Guideline Certified for Console Port Shutdown and Chassis Full Security Made in USA |
| --- | --- |

DYMEC – Temperature Hardened, American Made, Ruggedized, NSA Certified Endpoint, Industrial Ethernet Switches / Routers for SCADA, ITS, Transportation, Automation, Power Utilities, Edge Computing, AI and Industrial Control Systems. Made in USA. Cyber-Secure, TAA Compliant. Uses SFP’s / BiDi SFP’s / Mini Gbic & VDSL2 Industrial SFP for Long Range Ethernet.