KY-MSG2404 / DYMEC G Series

16+8+4 Port Industrial, Managed, Temperature Hardened, Gigabit, Rackmount,   Combo Ethernet Switch with ERPS & NSA Certified Endpoint

|  | - 16 x Gigabit Ethernet RJ-45 - 8 x Gigabit combo Ethernet (SFP & RJ-45) - 4 x 1000 SFP Slots - NSA Network Endpoint Guideline Certified for Console Port Shutdown and Chassis Full Security (Patent Pending) - Harsh environment & wide temperature range - RSTP, MSTP, ERPS, LACP for Network Redundancy - QoS CoS/DSCP to increase determinism - Supports IGMP Snooping v1 / v2, up to 1023 groups - DHCP Relay Option 82 for policies IP - SNMP v1/v2c/v3 for network management - USB storage for configuration-free replacement - Supports protocols Ethernet/IP and Modbus TCP - MAC Table for Troubleshooting |
| --- | --- |
| Specifications |  |

| Security | 802.1X, RADIUS, TACACS+, SSL, SSH |
| --- | --- |
| Management | IPv4, IPv6, Web, Telnet, Serial Console, CLI, uPnP, SNMP v1/v2c/v3 |
| Diagnostics | RMON, SNMP Inform/Trap, Syslog, SMTP, Port Mirroring, LLDP |
| Configuration | DHCP Server/Client, Relay Option 82, TFTP, USB |
| Filter | 802.1Q VLAN, Q-in-Q, IGMP Snooping v1/v2, IGMP Querier, Independent Vlan Learning, MAC Table |
| Time Sync. | NTP |
| QoS | CoS, DSCP, WRR/SPQ Queuing, Storm Protection |
| Industrial Profiles | Ethernet / IP, Modbus TCP |
| Interface |  |
| RJ45 Port | 16 x 10/100/1000T(x), auto negotiation speed duplex mode, auto MDI/MDI-X |
| Combo Port | 8 x 10/100/1000T(x) with SFP combo port, auto media detect |
| Fiber Port | 4 x 1000 SFP Slots |
| LED Indicators | Per unit: PWR1, PWR2, Fault, Ring Master, Ring State Ports: Link/Active with highest speed(Green), low speed(Amber) |
| Alarm Contact | 1 x relay output with current carrying capacity of 1A @ 24Volts DC |

| Technology |  |
| --- | --- |
| Standards | IEEE 802.3 10BaseT IEEE 802.3u 100BaseTX IEEE 802.3ab 1000BaseT IEEE 802.3z 1000BaseX IEEE 802.3x Flow Control IEEE 802.3ad LACP IEEE 802.1D Spanning Tree Protocol IEEE 802.1w Rapid STP IEEE 802.1s Multiple STP* ITU-T G.8032 / Y.1344 ERPS IEEE 802.1Q VLAN Tagging IEEE 802.1ad Stacked VLANs IEEE 802.1p Class of Service IEEE 802.1X Network Authentication IEEE 802.1AB LLDP |
| Processing Type | Store and Forward |
| Switch Properties |  |
| Priority Queues | 8 Queues |
| Jumbo Frame | 16000bytes |
| MAC Table Size | 16K |
| VLAN Table Size | 4094 |
| IGMP Group | 1023 |
| Packet Buffer | 12Mbits |
| Software Features |  |
| Network Redundancy | RSTP, MSTP, G.8032 ERPSv2, Port Trunking with LACP |

| Digital Inputs | 1 x isolated input from the electronics +13 to +30V for state "1" -30 to +3V for state "0" Max. input current: 8mA |  |
| --- | --- | --- |
| Button | Multiple function reset button |  |
| Serial Console | 1 x RS232 in RJ45 connector with console cable, 115.2Kbps, 8N1 |  |
| Storage | 1 x USB 2.0 storage for firmware update, configuration backup, restore, boot up and syslog |  |
| Power Requirements |  |  |
| Operation Voltage | 90~264 Volts AC or 12~48 Volts DC, redundant dual power supplies |  |
| Connection | 1 x 4-contact terminal block |  |
| Power Consumption | 30 Watts |  |
| Protection | Overload Current Protected, Reverse Polarity Protected |  |
| Mechanical Construction |  |  |
| Enclosure | Aluminum / Steel - Military Grade |  |
| Protection Class | IP30 |  |
| Dimensions | 440 x 44 x 331 mm (W x H x D) |  |
| Weight | 6.00 kg |  |
| Mounting | DIN-Rail Mounting, Optional Wall Mounting Kits |  |

| Environmental Limits |  |
| --- | --- |
| Operating Temperature | Standard: -40°C ~ 80°C (-40°F ~ 176°F) |
| Storage Temperature | -40°C ~ 85°C (-40°F ~ 185°F) |
| Ambient Relative Humidity | 5 to 95%, (Non-Condensing) |
| Regulatory Approvals |  |
| Altitude – GB4943.1-2011 | 0 ~ 5000 Meters Certified |
| EMI | FCC Part 15 Subpart B Class A CE EN 55022 Class A |
| EMS | IEC61000-4-2 (ESD Level 4) IEC61000-4-3 (RS Level 3) IEC61000-4-4 (EFT Level 4) IEC61000-4-5 (Surge Level 4) IEC61000-4-6 (CS Level 3) IEC61000-4-8 (Magnetic Field Level 4) |
| Free Fall | IEC60068-2-32 |
| Shock | IEC60068-2-27 |
| Vibration | IEC60068-2-6 |
| Green | RoHS Compliant |
| Certifications | UL 60950-1/62368 / IL / FCC / IEC / NEMA TS-2 / IETF / IEEE / ITUT |
| MTBF MIL-HDBK-217 IEC 62380 | >150,000 hours |
| Warranty TAA Compliant | 5 Years / Limited Lifetime Certified TAA Compliant * |

Dimensions (unit = mm)

| Ordering Information |  |  |
| --- | --- | --- |
| KY-MSG2404-AC | 16+8+4 Port Industrial Managed Gigabit Ethernet Switch - 1 6x 10/100/1000T(x) + 8 x Combo Ports + 4 x 1000 SFP Slots - Operating Temperature: -40°C ~ 85°C (-40°F ~ 185°F) - 2 x AC power inputs NSA Guideline Certified for Console Port Shutdown and Chassis Full Security |
| KY-MSG2404-AD | 16+8+4 Port Industrial Managed Gigabit Ethernet Switch - 16 x 10/100/1000T(x) + 8 x Combo Ports + 4 x 1000 SFP Slot - Operating Temperature: -40°C ~ 85°C (-40°F ~ 185°F) - 1 x AC power and 1 x 12-48 Volts DC power inputs NSA Guideline Certified for Console Port Shutdown and Chassis Full Security |
| KY-MSG2404-DC | 16+8+4 Port Industrial Managed Gigabit Ethernet Switch - 16 x 10/100/1000T(x) + 8 x Combo Ports + 4 x 1000 SFP Slots - Operating Temperature: -40°C ~ 85°C (-40°F ~ 185°F) - 2 x 12-48 Volts DC robust isolation power inputs NSA Guideline Certified for Console Port Shutdown and Chassis Full Security DYMEC – Temperature Hardened, American Made, Ruggedized, NSA Certified Endpoint, Industrial Ethernet Switches / Routers for SCADA, ITS, Transportation, Automation, Power Utilities, Edge Computing, AI and Industrial Control Systems. Made in USA. Cyber-Secure Copyright DYMEC 2018 – All Rights Reserved |