KY-MTX0804 / DYMEC MTG Series

8+4 Port, Industrial, Managed, PoE+ Booster, Ethernet Switch with 10 GbE Uplinks, Low Power / NSA Endpoint Certified with ERPsv2

|  | - 8 x Gigabit Ethernet ports with IEEE 802.3at PoE+ (30 Watt) - 4 x 1 / 10 Gigabit Ethernet SFP+ Slots - Built for harsh environments and wide temperature range applications - G. 8032 ERPsv2, RSTP, MSTP, ERPS, LACP for Network Redundancy - QoS CoS/DSCP to increase determinism - Supports IGMP Snooping v1/v2/v3, up to 1023 groups - DHCP Relay Option 82 for policies IP address assignment - SNMP v1/v2c/v3 for network management - Supports Storm Protection - USB storage for configuration-free replacement - Support industrial automation protocols Ethernet/IP and Modbus TCP - Supports PoE PD Alive Check Function for PoE Device Power Recycle - Power Boost for PoE+ - 5 Year Warranty |
| --- | --- |
| Specifications |  |

| Technology |  |
| --- | --- |
| Standards | IEEE 802.3 10BaseT IEEE 802.3u 100BaseTX IEEE 802.3ab 1000BaseT IEEE 802.3ae 10GBase over Fiber IEEE 802.3x Flow Control IEEE 802.3af Power over Ethernet IEEE 802.3at PoE+ IEEE 802.3ad/802.1AX LACP IEEE 802.1D Spanning Tree Protocol IEEE 802.1w Rapid STP IEEE 802.1s Multiple STP ITU-T G.8032 / Y.1344 ERPS IEEE 802.1Q VLAN Tagging IEEE 802.1ad Stacked VLANs IEEE 802.1p Class of Service IEEE 802.1X Network Authentication IEEE 802.1AB LLDP IEEE 1588 PTP |
| Processing Type | Store and Forward |
| Switch Properties |  |
| Priority Queues | 8 Queues |
| Jumbo Frame | 9216bytes |
| MAC Table Size | 16K |
| VLAN Table Size | 4094 |
| IGMP Group | 1023 |
| Packet Buffer | 12Mbits |
| Software Features |  |
| Network Redundancy | RSTP, MSTP, G.8032 ERPSv2, Port Trunking with LACP |
| Security | 802.1X, RADIUS, TACACS+, SSL, SSH |
| Management | IPv4, IPv6, Web, Telnet, Serial Console, CLI, SNMP v1/v2c/v3, uPnP |
| Diagnostics | RMON, SNMP Inform/Trap, Syslog, SMTP, Port Mirroring, LLDP |
| Configuration | DHCP Server/Client, Relay Option 82, TFTP, USB |
| Filter | 802.1Q VLAN, Q-in-Q, IGMP Snooping v1/v2/v3, IGMP Querier, Independent Vlan Learning |
| Time Sync. | NTP, IEEE1588v2 OC (Software Timestamp) |
| QoS | CoS, DSCP, WRR/SPQ Queuing, Storm Protection |
| PoE | Powering Status, PoE Monitoring, Scheduling, PD Alive Check |
| Industrial Profiles | Ethernet/IP, Modbus TCP |
| Interface |  |
| RJ45 Port | 8 x 10/100/1000T(x) with PoE+, auto negotiation speed duplex mode, auto MDI/MDI-X |
| Fiber Port | 4 x 1/10 Gigabit Ethernet SFP+ Slots |
| PoE Pin Out | V+, V+, V-, V-, for pin 1, 2, 3, 6, End-span, MDI Mode A |
| LED Indicators | Per unit: PWR1, PWR2, Fault, Ring Master, Ring State Ports: Link/Active with highest speed(Green), low speed(Amber) PoE: Output Power |
| Alarm Contact | 1 x relay output with current carrying capacity of 1A @ 24 Volts DC |
| Digital Inputs | 1 x isolated input from the electronics. +13 to +30 Volts for state "1" -30 to +3 Volts for state "0" Maximum input current: 8mA |
| Button | Multiple functions reset button |
| Serial Console | 1 x RS232 in RJ45 connector with console cable, 115.2Kbps, 8N1 |
| Storage | 1 x USB 2.0 storage for firmware update, configuration backup, restore, boot up and syslog |
| Power Requirements |  |
| Operation Voltage | 12-57 Volts DC, redundant dual inputs, >50 Volts DC for PoE+ output recommended |
| Connection | 1 x removable 4-contact terminal block |
| Power Consumption | 0.90A@24 Volts DC without PDs' consumption |
| PoE Power Budget | Maximum 30 Watts per PoE port. Maximum total output power for PD: 190 Watts @24 Volts input, 90 Watts @12 Volts input. |
| Protection | Overload Current Protected, Reverse Polarity Protected |
| Mechanical Construction |  |
| Enclosure | Military Grade – Structural Aluminum |
| Protection Class | IP30 |
| Dimensions | 72 x 145 x 118 mm (W x H x D) |
| Weight | 1.04 kg |
| Mounting | DIN-Rail Mounting, Optional Wall Mounting Kits |
| Environmental Limits |  |
| Operating Temperature | Extended: -40°C ~ 80°C (-40°F ~ 176°F) |
| Storage Temperature | -40°C ~ 85°C (-40°F ~ 185°F) |
| Ambient Relative Humidity | 5 to 95%, (Non-Condensing) |
| Regulatory Approvals |  |
| Altitude – GB4943.1-2011 | 0 ~ 5000 Meters |
| EMI | CE EN 55032 Class A |
| EMS | IEC61000-4-2 (ESD) IEC61000-4-3 (RS) IEC61000-4-4 (EFT) IEC61000-4-5 (Surge) IEC61000-4-6 (CS) IEC61000-4-8 (Magnetic Field) |
| Free Fall | IEC60068-2-32 |
| Shock | IEC60068-2-27 |
| Vibration | IEC60068-2-6 |
| Green | RoHS Compliant |
| Certifications | UL 60950-1/62368 IEC 61000-6-2 / FCC / UL / ITUT IEC 61000-6-4 / NEMA TS-2 EN 50121-4 |
| MTBF | >100,000 hours |
| Warranty TAA Compliant | 5 Years / Limited Lifetime Certified TAA Compliant |

Note from DYMEC Development. In a continuing effort of improvement and updated technology, product specifications are subject to change without notice.

Dimensions (unit = mm)

DYMEC Supports: 2 & 4 Wire LONG RANGE ETHERNET / VDSL2 SFP Modem

With long reach coverage, high noise immunity, and compact board size, DYMEC KY-Pro-180-CRD Ruggedized SFP modem is an ideal upgrade for your Enterprise or Industry applications. Used in Pairs.

##### VDSL2 – Long Range Ethernet Application (KY-Pro-180-CRD)

DYMEC KY-SFP-T

Copper SFP Transceiver – Used to Add Additional RJ-45 Copper Ports to your KY-3170 Series Switch

Easy to Install & Configure – GUI Selectable

| Ordering Information |  |
| --- | --- |
| KY-MTX0804 | 8+4 Port Industrial Managed PoE+ Booster Ethernet Switch with 1 / 10 GbE Uplinks - 8x 10/100/1000T(x) with 30W PSE + 4 x 1/10 Gigabit Ethernet SFP+ Slots - Operating Temperature: -40°C ~ 80°C (-40°F ~ 176°F) NSA Guideline Certified for Console Port Shutdown and Chassis Full Security |

DYMEC – Temperature Hardened, American Made, Ruggedized, NSA Certified Endpoint, Industrial Ethernet Switches / Routers for SCADA, ITS, Transportation, Automation, Power Utilities, Edge Computing, AI and Industrial Control Systems. Made in USA. Cyber-Secure, TAA Compliant. Uses SFP’s / BiDi SFP’s / Mini Gbic & VDSL2 Industrial SFP for Long Range Ethernet.