Industrial Managed Ethernet Switch User Manual – Layer 3

_______________________________________________________________________________

Firmware Version 1.1.72 User Manual Compliant: For Military contracts that require the continuous monitoring and protection of industrial control systems (ICS) Networks Operating / monitoring the operating utilities such as Water, Sewer, Electrical, Security, Video, Building Controls, Street, and Intelligent Transportation Systems (ITS). Uses: Cyber-Lock, IP Device Binding, Cyber-Secure Video, Clean Code Technology, & Secure Console / Chassis (Patent Pending) Meets NSA Guidelines for Secure Network Endpoints

Models

Copyright DYMEC 2020 – All Rights Reserved

| DYMEC Models Covered by This Manual |  |
| --- | --- |
| All Layer 3 Series: | KY-CSxxxx, KY-CPxxxx, KY-CTxxxx |  |

Content

VLAN IP3

Static Route5

OSPFv28

RIPv1/v215

VRRP21

# VLAN IP

We usually configure VLANs to divide broadcast domains in the Local Area Network (LAN). To communicate with others in another VLAN, we need the device to support inter-VLAN routing. Users can use VLANs to create Layer 3 interfaces by assigning IP address in different subnets to the VLAN. Before configuring any kind of routing protocols, users have to make sure the VLAN IP Address is configured.

Configure VLAN IP Address - WEB

- DNS Server 1 / 2

Specifies the IP address of the DNS Server so that the users can connect to another device based on the URL instead of the IP address.

The default DNS Server 1 is "8.8.8.8". It is provided by Google.

The default DNS Server 2 is BLANK.

- Interface Settings

  - VLAN ID

Users have to create VLAN in the VLAN Settings Page before configure its IP Address.

  - IP Address

Assign a unique static IP Address for the VLAN ID.

The default IP Address for VLAN 1 is "192.168.10.1".

  - Subnet Mask

Define the type of network, to which this device is connected to.

The default Subnet Mask for VLAN 1 is "255.255.255.0".

- (Apply Button)

After configuring above fields, click "Apply" button to make the changes effective.

Configure VLAN IP Address - CLI

- Configure VLAN IP Address (VLAN Mode)

Cmd: ip address [IP_ADDR] [NETMASK]

- Display VLAN IP Address (VLAN Mode)

Cmd: show ip address

- Remove VLAN IP Address (VLAN Mode)

Cmd: no ip address

- Display All Configured VLAN IP Address (Config Mode)

Cmd: show vlan ip table

# Static Route

Static Route is the most basic routing method. Users have to manually-configure routing entries into routing table. Static Route is fixed and unchanged if the Network is changed. Static Route is best to be configured at a gateway of last resort. The Network administrator has the full control over the static routing behavior, so the administrator must have enough background knowledge to fully understand and handle the configuration.

Configure Static Route - WEB

- IP Address

Assign the IP Address to configure the static routing path.

- Netmask

The Netmask is a number mapping to the Subnet Mask. For example, users configure the Netmask to 24 implies to configure the Subnet Mask to 255.255.255.0.

- Next Hop

The Next Hop is an IP Address for a VLAN ID. The Next Hop implies the next node that the packet will be transferred to.

- (Apply Button)

After configuring above fields, click "Apply" button to make the changes effective.

Configure Static Route - CLI

All the following commands are under config mode

- Add Static Route

Cmd: ip route [IP_ADDR] [NETMASK:0-32] [NEXT_HOP]

- Display Static Route

Cmd: show ip route static

- Delete Static Route

Cmd: no ip route [IP_ADDR] [NETMASK:0-32]

Static Route Example

In the following diagram, we are going to configure a topology with Static Route for the system.

- Configuration for Device RA

| Command | Description |
| --- | --- |
| Switch> enable | Enter “Privileged mode” |
| Switch# configure terminal | Enter “Configuration mode” |
| Switch(config)# vlan 1 | Enter “VLAN mode” for VLAN 1 |
| Switch(config-vlan1)# ip address 192.168.11.182 255.255.255.0 | Configure IP Address to “192.168.11.182” for VLAN 1 |
| Switch(config-vlan1)# vlan 2 | Enter “VLAN mode” for VLAN 2 |
| Switch(config-vlan2)# ip address 192.168.12.182 255.255.255.0 | Configure IP Address to “192.168.12.182” for VLAN 2 |
| Switch(config-vlan2)# member tag 11 | Add Lan11 to VLAN 2 with tagged |
| Switch(config-vlan2)# exit | Exit “VLAN mode”, back to “Configuration mode” |
| Switch(config)# ip route 192.168.10.0 24 192.168.12.166 | Configure static router for subnet 192.168.10.0/24. The next hop is 192.168.12.166 |

- Configuration for Device RB

| Command | Description |
| --- | --- |
| Switch> enable | Enter “Privileged mode” |
| Switch# configure terminal | Enter “Configuration mode” |
| Switch(config)# vlan 1 | Enter “VLAN mode” for VLAN 1 |
| Switch(config-vlan1)# ip address 192.168.10.166 255.255.255.0 | Configure IP Address to “192.168.10.166” for VLAN 1 |
| Switch(config-vlan1)# vlan 2 | Enter “VLAN mode” for VLAN 2 |
| Switch(config-vlan2)# ip address 192.168.12.166 255.255.255.0 | Configure IP Address to “192.168.12.166” for VLAN 2 |
| Switch(config-vlan2)# member tag 8 | Add Lan8 to VLAN 2 with tagged |
| Switch(config-vlan2)# exit | Exit “VLAN mode”, back to “Configuration mode” |
| Switch(config)# ip route 192.168.11.0 24 192.168.12.182 | Configure static router for subnet 192.168.11.0/24. The next hop is 192.168.12.182 |

# OSPFv2

Open Shortest Path First (OSPF) is one of the dynamic routing protocols and OSPFv2 is for IPv4 Networks. OSPF is the most widely used interior Gateway Protocol (IGP) and it is designed for medium-and-large-scale enterprise networks. OSPF nodes exchange neighbors’ information by Link-State and calculate the shortest path with their Cost to transmit packets. OSPF is operating under a hierarchical system and divide the system to several Areas by different Area IDs. OSPF nodes only exchange information with the nodes in the same Area.

Configure OSPFv2 – WEB

For more information, hover the mouse over the icon in the system.

- OSPF Mode

“Enable” or “Disable” OSPFv2 Function. The OSPFv2 function is default disabled.

Please make sure the status of RIP is Disabled before Enable OSPFv2 function.

- Networks

  - Network ID

The Networks ID implies the first IP Address of the subnet that will join the OSPF routing.

  - Prefix

The Prefix is a number mapping to the Subnet Mask. For example, users configure the Prefix to 24 implies to configure the Subnet Mask to 255.255.255.0.

The range of Prefix is from 0 to 32.

  - Area

The Area is an identify and only the nodes in the same area will exchange the OSPF information with each other. The Area 0 is a special area, and also called Backbone Area. All other areas must connect to the backbone area.

The range of Area is from 0 to 4294967295.

: Click the plus icon to add an OSPF entry.

: Click the remove icon to delete the OSPF entry.

- Router-ID

The Router-ID implies the identity for the working OSPFv2.

The format of Router-ID is A.B.C.D.

- Hello Interval

The Hello Interval is the period to send “Hello Message” to OSPF neighbors.

The range of Hello Interval is from 1 to 65535 second(s).

The default Hello Interval is 10 seconds.

- Metric

The Metric is the total cost of the path from source device to destination device.

The range of Metric is from 1 to 65535.

The default Metric is 10.

- (Apply Button)

After configuring above fields, click "Apply" button to make the changes effective.

OSPFv2 Status

- OSPF Mode

Display the state (“Enable” or “Disable”) of OSPF function.

- Local IP

Display the information that configured on the localhost for OSPFv2.

- Neighbor Information

Display the status of the neighborhood.

- Neighbor IP

Display the information that learned from OSPF neighbors.

- Hello Interval

Display the Hello Interval value of designated VLAN.

- Metric

Display the Metric of designated VLAN.

Configure OSPFv2 - CLI

All the following commands are under Config Mode

- Enable OSPF Function

Cmd: router ospf enable

- Disable OSPF Function

Cmd: no router ospf

- Display OSPF Running State

Cmd: show router ospf mode

- Configure OSPF Router ID

Cmd: router ospf router-id [ROUTER_ID]

- Display OSPF Router ID

Cmd: show router ospf router-id

- Clear OSPF Router ID

Cmd: no router ospf router-id

- Configure OSPF Network

Cmd: router ospf network [IP_ADDR] [NETMASK:0-32] [AREA_ID]

- Display OSPF Network

Cmd: show router ospf network

- Remove OSPF Network Rule for Specific Subnet

Cmd: no router ospf network [IP_ADDR] [NETMASK:0-32]

- Enable OSPF Authentication with Text on Specific Area

Cmd: router ospf authentication area [AREA_ID]

- Enable OSPF Authentication with MD5 on Specific Area

Cmd: router ospf authentication message-digest area [AREA_ID]

- Display OSPF Authentication State

Cmd: show router ospf authentication

- Disable OSPF Authentication on Specific Area

Cmd: no router ospf authentication area [AREA_ID]

- Display OSPF Operation Status

Cmd: show router ospf

- Display OSPF Neighbors’ Status

Cmd: show router ospf neighbor

All the following commands are under VLAN Mode

- Configure OSPF Hello Interval

Cmd: ip ospf hello-interval [1-65535]

The default OSPF Hello Interval is 10 seconds.

- Display OSPF Hello Interval

Cmd: show ip ospf hello-interval

- Clear OSPF Hello Interval

Cmd: no ip ospf hello-interval

- Configure OSPF Dead Interval

Cmd: ip ospf dead-interval [1-65535]

The default OSPF Dead Interval is 4 times hello interval.

- Display OSPF Dead Interval

Cmd: show ip ospf dead-interval

- Clear OSPF Dead Interval

Cmd: no ip ospf dead-interval

- Enable OSPF Authentication with Text on Specific VLAN

Cmd: ip ospf authentication

- Enable OSPF Authentication with MD5 on Specific VLAN

Cmd: ip ospf authentication message-digest

- Display OSPF Authentication State

Cmd: show ip ospf authentication

- Disable OSPF Authentication on Specific VLAN

Cmd: no ip ospf authentication

- Configure OSPF Authentication Password

Cmd: ip ospf authentication-key [PASSWD]

- Display OSPF Authentication Password

Cmd: show ip ospf authentication-key

- Clear OSPF Authentication Password

Cmd: no ip ospf authentication-key

- Configure OSPF MD5 Authentication Password

Cmd: ip ospf message-digest-key [KEY_ID] md5 [PASSWD]

- Display OSPF MD5 Authentication Password

Cmd: show ip ospf message-digest-key

- Clear OSPF MD5 Authentication Password

Cmd: no ip ospf message-digest-key [KEY_ID]

OSPFv2 Example

In the following diagram, we are going to configure a topology with Dynamic Routing – OSPFv2 for the system. Use the following commands to check the link state after configurations are completed:

  - show router ospf

  - show router ospf neighbor

  - show ip route

*Simulation Internet through RC.*

- Configuration for RA

| Command | Description |
| --- | --- |
| Switch> enable | Enter “Privileged mode” |
| Switch# configure terminal | Enter “Configuration mode” |
| Switch(config)# vlan 1 | Enter “VLAN mode” for VLAN 1 |
| Switch(config-vlan1)# ip address 192.168.11.182 255.255.255.0 | Configure IP Address to “192.168.11.182” for VLAN 1 |
| Switch(config-vlan1)# vlan 6 | Enter “VLAN mode” for VLAN 6 |
| Switch(config-vlan6)# ip address 192.168.6.182 255.255.255.0 | Configure IP Address to “192.168.6.182” for VLAN 6 |
| Switch(config-vlan6)# member tag 4 | Add Lan4 to VLAN6 with tagged |
| Switch(config-vlan6)# exit | Exit “VLAN mode”, back to “Configuration mode” |
| Switch(config)# router ospf router-id 0.0.0.1 | Configure OSPF router id to 0.0.0.1 |
| Switch(config)# router ospf network 192.168.11.0 24 0 | Configure subnet 192.168.11.0/24 to join OSPF network area 0 |
| Switch(config)# router ospf network 192.168.6.0 24 0 | Configure subnet 192.168.6.0/24 to join OSPF network area 0 |
| Switch(config)# router ospf enable | Enable OSPFv2 function on RA |

- Configuration for RB

| Command | Description |
| --- | --- |
| Switch> enable | Enter “Privileged mode” |
| Switch# configure terminal | Enter “Configuration mode” |
| Switch(config)# vlan 1 | Enter “VLAN mode” for VLAN 1 |
| Switch(config-vlan1)# ip address 192.168.10.166 255.255.255.0 | Configure IP Address to “192.168.10.166” for VLAN 1 |
| Switch(config-vlan1)# vlan 7 | Enter “VLAN mode” for VLAN 7 |
| Switch(config-vlan7)# ip address 192.168.7.166 255.255.255.0 | Configure IP Address to “192.168.7.166” for VLAN 7 |
| Switch(config-vlan7)# member tag 6 | Add Lan6 to VLAN 7 with tagged |
| Switch(config-vlan7)# exit | Exit “VLAN mode”, back to “Configuration mode” |
| Switch(config)# router ospf router-id 0.0.0.2 | Configure OSPF router id to 0.0.0.2 |
| Switch(config)# router ospf network 192.168.10.0 24 0 | Configure subnet 192.168.10.0/24 to join OSPF network area 0 |
| Switch(config)# router ospf network 192.168.7.0 24 0 | Configure subnet 192.168.7.0/24 to join OSPF network area 0 |
| Switch(config)# router ospf enable | Enable OSPFv2 function on RB |

- Configuration for RC

| Command | Description |
| --- | --- |
| Switch> enable | Enter “Privileged mode” |
| Switch# configure terminal | Enter “Configuration mode” |
| Switch(config)# vlan 1 | Enter “VLAN mode” for VLAN 1 |
| Switch(config-vlan1)# ip address 192.168.3.108 255.255.255.0 | Configure IP Address to “192.168.3.108” for VLAN 1 |
| Switch(config-vlan1)# vlan 6 | Enter “VLAN mode” for VLAN 6 |
| Switch(config-vlan6)# ip address 192.168.6.108 255.255.255.0 | Configure IP Address to “192.168.6.108” for VLAN 6 |
| Switch(config-vlan6)# member tag 8 | Add Lan8 to VLAN6 with tagged |
| Switch(config-vlan6)# vlan 7 | Enter “VLAN mode” for VLAN 7 |
| Switch(config-vlan7)# ip address 192.168.7.108 255.255.255.0 | Configure IP Address to “192.168.7.108” for VLAN 7 |
| Switch(config-vlan7)# member tag 4 | Add Lan4 to VLAN 7 with tagged |
| Switch(config-vlan7)# exit | Exit “VLAN mode”, back to “Configuration mode” |
| Switch(config)# router ospf router-id 0.0.0.3 | Configure OSPF router id to 0.0.0.3 |
| Switch(config)# router ospf network 192.168.6.0 24 0 | Configure subnet 192.168.6.0/24 to join OSPF network area 0 |
| Switch(config)# router ospf network 192.168.7.0 24 0 | Configure subnet 192.168.7.0/24 to join OSPF network area 0 |
| Switch(config)# router ospf enable | Enable OSPFv2 function on RC |

# RIPv1/v2

Routing Information Protocol (RIP) is another dynamic routing protocol for a small-scale Network. RIP uses Distance Vector to determine the routing path. The best path for RIP is the least hops during transmission. This is also the weakness of RIP because the path with least hops might not be the fastest path.

The RIP members broadcast their routing table to each other every 30 seconds to synchronize the Network and maintain the routing paths to be the least hops. The max number of hops that can be transferred between two nodes (routers) is 15. This is why the RIP function is only suitable for a small Network.

Configure RIPv1/v2 - WEB

- RIP Mode

“Enable” or “Disable” RIP Function. The RIP function is default disabled.

Please make sure the status of OSPFv2 is Disabled before Enable RIP function.

- Networks

  - Network ID

The Networks ID implies the first IP Address of the subnet that will join the RIP routing.

  - Prefix

The Prefix is a number mapping to the Subnet Mask. For example, users configure the Prefix to 24 implies to configure the Subnet Mask to 255.255.255.0.

The range of Prefix is from 0 to 32.

- Version

Configure the Version of RIP. We support RIPv2 by default but users can configure the version to support RIPv1.

- Timers

  - Update Time

The Update Time is the interval to send broadcast for updating routing table with other routers in the same RIP Network.

The range of Update Time is from 5 to 2147483647.

The default Update Time is 30 seconds.

  - Timeout

The Timeout value is also called Aging Time. It will be count down from configured value until 0. If the system received a packet with the same RIP path, the Aging Time will be reset to the initial configured value and count down again. If the Aging Time is counting down to 0, the routing entry will be deleted from the routing table.

The range of Timeout is from 5 to 2147483647.

The default Timeout is 180 seconds.

  - Garbage

The Garbage-Collection Timer is used for a newly created routing path. When a new routing path is built, the timer creates and counts down. The routing path will be discarded if the timer is 0.

The range of Garbage is from 5 to 2147483647.

The default Garbage is 120 seconds.

RIPv1/v2 Status

- RIP Status

Display the state (“Enable” or “Disable”) of RIP function.

- Local IP

Display the information that configured on the localhost for RIP.

- Routing Network Information

Display the routing information that exchanges with other routers.

Configure RIPv1/v2 - CLI

All the following commands are under Config Mode

- Enable RIP Function

Cmd: router rip enable

- Disable RIP Function

Cmd: no router rip

- Display RIP Running State

Cmd: show router rip mode

- Configure RIP Neighbor IP Address

Cmd: router rip neighbor [IP_ADDR]

- Remove RIP Neighbor IP Address

Cmd: no router rip neighbor [IP_ADDR]

- Configure RIP Network

Cmd: router rip network [IP_ADDR] [NETMASK: 0-32]

- Display RIP Network

Cmd: show router rip network

- Remove RIP Network Rule for Specific Subnet

Cmd: no router rip network [IP_ADDR] [NETMASK: 0-32]

- Configure RIP Timers

Cmd: router rip timers [UPDATE_TIMER] [TIMEOUT] [GARBAGE]

Note: the range of timers is [5- 2147483647]

- Clear RIP Timers

Cmd: no router rip timers

- Configure RIP Version

Cmd: router rip version [1|2]

- Clear RIP Version

Cmd: no router rip version

- Display RIP Routing Table

Cmd: show router rip state

- Display RIP Operation Status

Cmd: show router rip status

RIP Example

In the following diagram, we are going to configure a topology with Dynamic Routing – RIP for the system. Use the following commands to check the link state after configurations are completed:

  - show router rip status

  - show router rip state

  - show ip route

*Simulation Internet through RC.*

- Configuration for RA

| Command | Description |
| --- | --- |
| Switch> enable | Enter “Privileged mode” |
| Switch# configure terminal | Enter “Configuration mode” |
| Switch(config)# vlan 1 | Enter “VLAN mode” for VLAN 1 |
| Switch(config-vlan1)# ip address 192.168.11.182 255.255.255.0 | Configure IP Address to “192.168.11.182” for VLAN 1 |
| Switch(config-vlan1)# vlan 6 | Enter “VLAN mode” for VLAN 6 |
| Switch(config-vlan6)# ip address 192.168.6.182 255.255.255.0 | Configure IP Address to “192.168.6.182” for VLAN 6 |
| Switch(config-vlan6)# member tag 4 | Add Lan4 to VLAN6 with tagged |
| Switch(config-vlan6)# exit | Exit “VLAN mode”, back to “Configuration mode” |
| Switch(config)# router rip network 192.168.11.0 24 0 | Configure subnet 192.168.11.0/24 to join RIP network |
| Switch(config)# router rip network 192.168.6.0 24 0 | Configure subnet 192.168.6.0/24 to join RIP network |
| Switch(config)# router rip enable | Enable RIP function on RA |

- Configuration for RB

| Command | Description |
| --- | --- |
| Switch> enable | Enter “Privileged mode” |
| Switch# configure terminal | Enter “Configuration mode” |
| Switch(config)# vlan 1 | Enter “VLAN mode” for VLAN 1 |
| Switch(config-vlan1)# ip address 192.168.10.166 255.255.255.0 | Configure IP Address to “192.168.10.166” for VLAN 1 |
| Switch(config-vlan1)# vlan 7 | Enter “VLAN mode” for VLAN 7 |
| Switch(config-vlan7)# ip address 192.168.7.166 255.255.255.0 | Configure IP Address to “192.168.7.166” for VLAN 7 |
| Switch(config-vlan7)# member tag 6 | Add Lan6 to VLAN 7 with tagged |
| Switch(config-vlan7)# exit | Exit “VLAN mode”, back to “Configuration mode” |
| Switch(config)# router rip network 192.168.10.0 24 0 | Configure subnet 192.168.10.0/24 to join RIP network |
| Switch(config)# router rip network 192.168.7.0 24 0 | Configure subnet 192.168.7.0/24 to join RIP network |
| Switch(config)# router rip enable | Enable RIP function on RB |

- Configuration for RC

| Command | Description |
| --- | --- |
| Switch> enable | Enter “Privileged mode” |
| Switch# configure terminal | Enter “Configuration mode” |
| Switch(config)# vlan 1 | Enter “VLAN mode” for VLAN 1 |
| Switch(config-vlan1)# ip address 192.168.3.108 255.255.255.0 | Configure IP Address to “192.168.3.108” for VLAN 1 |
| Switch(config-vlan1)# vlan 6 | Enter “VLAN mode” for VLAN 6 |
| Switch(config-vlan6)# ip address 192.168.6.108 255.255.255.0 | Configure IP Address to “192.168.6.108” for VLAN 6 |
| Switch(config-vlan6)# member tag 8 | Add Lan8 to VLAN6 with tagged |
| Switch(config-vlan6)# vlan 7 | Enter “VLAN mode” for VLAN 7 |
| Switch(config-vlan7)# ip address 192.168.7.108 255.255.255.0 | Configure IP Address to “192.168.7.108” for VLAN 7 |
| Switch(config-vlan7)# member tag 4 | Add Lan4 to VLAN 7 with tagged |
| Switch(config-vlan7)# exit | Exit “VLAN mode”, back to “Configuration mode” |
| Switch(config)# router rip network 192.168.6.0 24 0 | Configure subnet 192.168.6.0/24 to join RIP network |
| Switch(config)# router rip network 192.168.7.0 24 0 | Configure subnet 192.168.7.0/24 to join RIP network |
| Switch(config)# router rip enable | Enable RIP function on RC |

# VRRP

Virtual Router Redundancy Protocol (VRRP) is a function to avoid single point of failure in the static default routed environment. There are two or more routers in a VRRP group to backup each other. They share the same virtual IP address in the Network and one of them called Master provides routing services for other devices. Others in the same VRRP group are Backup routers. When the Master router is broken, the Backup routers will replace the original Master router based on their Priority value. The router with the highest Priority value will be the next Master, so that the routing can be uninterrupted.

Configure VRRP – WEB

For more information, hover the mouse over the icon in the system.

- VLAN ID

Display the current VLAN ID that is configured with members and IP Address.

- Group ID

The Group ID is the identity for the VRRP Group. All routers in the same VRRP group MUST be configured with the same Group ID.

The range of Group ID is from 1 to 255

- Adv. Timer

The timer is the period to send Advertisement to make sure the Master is still alive. The Master route will be changed if other Backup routers miss Advertisement message after the timer expired to make sure the continuous services.

The range of Adv. Timer is from 1 to 60 seconds

- Priority

The Priority is used to decide the Master in the VRRP group.

The range of Priority is from 1 to 254

Note: the higher the value the higher the priority

- Virtual IP Address

The Virtual IP Address is a public IP Address in the Network. All routers in the same VRRP group are configured with the same Virtual IP Address.

Note: The Virtual IP Address has to be configured different from VLAN IP Addresses.

- Preempt

“Enable” or “Disable” Preempt Mode. When the Preempt Mode is disabled, other VRRP routers are not allowed to preempt the Master role.

- Mode

“Enable” or “Disable” VRRP function on this VLAN.

Note: Users have to configure all VRRP parameters before enable VRRP function.

- State

There are four states for the VRRP

Master: This is a major router that provides services in the VRRP group.

Backup: This is a backup router in the VRRP group. When the VRRP Master is broken, the backup router will provide services.

Init: VRRP function is under initialization.

NULL: VRRP function is disabled.

Configure VRRP – CLI

All the following commands are under Config Mode

- Display All VRRP Configuration Information

Cmd: show vrrp-config

All the following commands are under VLAN Mode

- Display All VRRP Configurations

Cmd: show vrrp all

- Disable VRRP and Remove All VRRP Configuration

Cmd: no vrrp all

- Set Advertise Interval Timer of VRRP

Cmd: vrrp adv-timer [1-60]

- Display Advertise Interval Timer of VRRP

Cmd: show vrrp adv-timer

- Disable VRRP and Remove Advertise Interval Timer of VRR

Cmd: no vrrp adv-timer

- Enable VRRP Function

Cmd: vrrp enable

- Display State of VRRP

Cmd: show vrrp state

- Disable VRRP

Cmd: no vrrp state

- Set Virtual Group ID of VRRP

Cmd: vrrp group-id [1-255]

- Display Virtual Group ID of VRRP

Cmd: show vrrp group-id

- Disable VRRP and Remove Virtual Group ID of VRRP

Cmd: no vrrp group-id

- Set Virtual Server IP of VRRP

Cmd: vrrp ip-address [Virtual IP address]

- Display Virtual IP of VRRP

Cmd: show vrrp ip-address

- Disable VRRP and Remove Virtual IP of VRRP

Cmd: no vrrp ip-address

- Enable Preemption Mode of VRRP

Cmd: vrrp preempt

- Display Preemption Mode of VRRP

Cmd: show vrrp preempt

- Disable Preemption Mode of VRRP

Cmd: no vrrp preempt

- Set Priority of VRRP

Cmd: vrrp priority [1-254]

- Display Priority of VRRP

Cmd: show vrrp priority

- Disable VRRP and Remove Priority of VRRP

Cmd: no vrrp priority