KY-MT1000G-M12 / DYMEC M-12 Series   10 Port, M12 Industrial, Managed PoE+, Super Booster Gigabit Ethernet Switch for Transportation, Railway & Trackside

|  | - Certified EN50155 - 10x 10/100/1000T(x) M12 connector with 8x IEEE 802.3at PoE+ - Built for harsh environments and wide temperature range application - RSTP, MSTP, ERPS, LACP for Network Redundancy - QoS CoS/DSCP to increase determinism - 5 Year / Limited Lifetime Warranty - Supports IGMP Snooping v1/v2/v3, up to 1023 groups - DHCP Relay Option 82 for policies IP address assignment - SNMP v1/v2c/v3 for network management - Supports Storm Protection - Support industrial automation protocols Ethernet/IP and Modbus TCP - Supports PoE PD Alive Check Function for PoE Device Power Recycle - Superpower Boost for PoE+ |
| --- | --- |
| Specifications |  |

| Technology |  |
| --- | --- |
| Standards | IEEE 802.3 10BaseT IEEE 802.3u 100BaseTX IEEE 802.3ab 1000BaseT IEEE 802.3x Flow Control IEEE 802.3af Power over Ethernet IEEE 802.3at PoE+ IEEE 802.3ad/802.1AX LACP IEEE 802.1D Spanning Tree Protocol IEEE 802.1w Rapid STP IEEE 802.1s Multiple STP ITU-T G.8032 / Y.1344 ERPS IEEE 802.1Q VLAN Tagging IEEE 802.1ad Stacked VLANs IEEE 802.1p Class of Service IEEE 802.1X Network Authentication IEEE 802.1AB LLDP |
| Processing Type | Store and Forward |
| Switch Properties |  |
| Priority Queues | 8 Queues |
| Jumbo Frame | 12000bytes |
| MAC Table Size | 16K |
| VLAN Table Size | 4094 |
| IGMP Group | 1023 |
| Packet Buffer | 12Mbits |
| Software Features |  |
| Network Redundancy | RSTP, MSTP, G.8032 ERPSv2, Port Trunking with LACP |
| Security | 802.1X, RADIUS, TACACS+, SSL, SSH |
| Management | IPv4, IPv6, Web, Telnet, Serial Console, CLI, SNMP v1/v2c/v3, uPnP |
| Diagnostics | RMON, SNMP Inform/Trap, Syslog, SMTP, Port Mirroring, LLDP |
| Configuration | DHCP Server/Client, Relay Option 82, TFTP |
| Filter | 802.1Q VLAN, Q-in-Q, IGMP Snooping v1/v2/v3, IGMP Querier, Independent Vlan Learning |
| Time Sync. | NTP |
| QoS | CoS, DSCP, WRR/SPQ Queuing, Storm Protection |
| PoE | Powering Status, PoE Monitoring, Scheduling, PD Alive Check |
| Industrial Profiles | Ethernet/IP, Modbus TCP |
| Interface |  |
| Network Connector | 10 x 10/100/1000T(x) M12 Female X-coded 8-pole with 8x PoE+, auto negotiation speed duplex mode, auto MDI/MDI-X |
| LED Indicators | Per unit: PWR1, PWR2, Status Ports: Link/Active with highest speed (Green), low speed (Amber) PoE: Output Power |
| Alarm Contact | 1 x relay output with current carrying capacity of 1A @ 24 VDC |
| Serial Console | 1 x RS232 in M12 connector with console cable, 115.2Kbps, 8N1 |
| Power Requirements |  |
| Operation Voltage | LV: 24-48 Volts DC, HV: 72-110 Volts DC, redundant dual inputs |
| Connection | M12 Male S-coded 4-pole |
| Power Consumption | <15 Watts without PDs' consumption |
| PoE Power Budget | Maximum 30 Watts per PoE port. Maximum total output power for PD: 190 Watts @24V input. |
| Protection | Overload Current Protected, Reverse Polarity Protected |
| Mechanical Construction |  |
| Enclosure | SECC |
| Protection Class | IP54 |
| Dimensions | 143 x 209 x 77 mm (W x H x D) |
| Weight | 1.90 kg |
| Mounting | Wall Mounting Kits, Optional DIN-Rail Mounting |
| Environmental Limits |  |
| Operating Temperature | Extended: -40°C ~ 85°C (40°F ~ 176°F) |
| Storage Temperature | -40°C ~ 85°C (40°F ~ 185°F) |
| Ambient Relative Humidity | 5 to 95%, (Non-Condensing) |
| Regulatory Approvals |  |
| EMI | FCC Part 15 Subpart B Class A CE EN 55032 Class A |
| EMS | IEC61000-4-2 (ESD) IEC61000-4-3 (RS) IEC61000-4-4 (EFT) IEC61000-4-5 (Surge) IEC61000-4-6 (CS) IEC61000-4-8 (Magnetic Field) |
| Free Fall | IEC60068-2-32 |
| Shock | IEC61373 |
| Vibration | IEC61373 |
| Green | RoHS Compliant |
| Certifications | EN 50155 EN 45545-2 |
| MTBF | >100,000 Hours |
| Warranty | 5 Years / Limited Lifetime |

Note from DYMEC Development In a continuing effort of improvement and updated technology, product specifications are subject to change without notice.

Dimensions (unit = mm)

| Ordering Information |  |
| --- | --- |
| KY-MT1000G-M12-LV | 10 Port M12 Industrial Managed PoE+ Booster Gigabit Ethernet Switch - 10 x 10/100/1000T(x) M12 connector with 8 x 30W PSE - Power Input: 24-48 Volts DC - Operating Temperature: -40°C ~ 85°C (40°F ~ 176°F) |
| KY-MT1000G-M12-HV | 10 Port M12 Industrial Managed PoE+ Booster Gigabit Ethernet Switch - 10 x 10/100/1000T(x) M12 connector with 8 x 30W PSE - Power Input: 72-110 Volts DC - Operating Temperature: -40°C ~ 85°C (40°F ~ 176°F) |